SAFETY DATA

SDS No 318

Issue 2   July 2004

GRANATECH

(ALL DERIVATIVES)

- IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY/UNDERTAKING

Product Name	Granatech (All derivatives)

Use	Granite based slurry pointing system

Company		EasiPoint Marketing Ltd

Address		Restoration House

Durmhead Road

Chorley North Industrial Estate

Chorley Lancashire PR6 7DE

Telephone	01257 224900

Fax		01257 224901

E-mail		enquiries@easipoint.co.uk

Web Site		www.easipoint.co.uk

- COMPOSITION/INFORMATION ON INGREDIENTS

Mixture of sands, fillers, surfactants, polymers and pigments not classified as hazardous. Portland Cement CAS 65997-15-1  EEC-Symbol Xi R Phrases 36/37/38, 41

- HAZARDS IDENTIFICATION

Irritating to eyes, respiratory system and skin.  Risk of serious damage to eyes.

- FIRST AID MEASURES

Eye Contact

Rinse immediately with water for at least 15 minutes.  If irritation persists get medical attention.

Skin Contact

Wash with soap and water.

Inhalation

Move to fresh air.

Ingestion

Wash mouth with plenty of water.  Do NOT induce vomiting.  In case of spontaneous vomiting ensure vomit can drain freely to avoid suffocation.  Get medical attention.

- FIRE FIGHTING MEASURES

Suitable Extinguishing Media

None required as not flammable

Exposure Hazards

None as not flammable

- ACCIDENTAL RELEASE MEASURES

# Personal Precautions

Wear suitable protective clothing, gloves and eye/face protection.

# Environmental Precautions

Prevent contamination of soil, drains and surface water.

# Methods For Cleaning

Take up material.  Avoid air borne dust.  Place in closable container and dispose of as waste.

- HANDLING AND STORAGE

# Handling

Ensure adequate ventilation.  Do not breathe dust.  Avoid eye contact.

# Storage

Keep in original container.  Keep Dry.

- EXPOSURE CONTROLS/PERSONAL PROTECTION

# Engineering Protective Measures

No special measure required.

# Occupational Exposure Limits

Amorphous Silica 8Hour TWA;

Total Inhalation Dust OES 6.0mg/m3

Respirable Dust OES 2.4mg/m3

Respitatory Protection

In case of insufficient ventilation wear suitable respiratory equipment.

Hand Protection

Wear suitable gloves (eg PVC)

Eye Protection

Wear suitable safety glasses to BSEN 166

Skin Protection

Wear overalls and closed footwear

- PHYSICAL AND CHEMICAL PROPERTIES

Melting Point/Range	Not Applicable

Boiling Point/Range	Not Applicable

Oxidising Properties	Not Applicable

Autoflammabilty		Not Applicable

Solubility	 In Water		Forms alkaline mixture

Vapour Pressure		Not Applicable

Partition Coefficient	Not Applicable

Explosive Properties	Not Applicable

Appearance	Grey coloured granular powder

Odour 			None

Density			Bulk Density 1.5-1.8 approx

Flashpoint		Not Applicable

Ignition			Not Applicable

Ph Value			Approximately 13 when in

aqueous mixture

Viscosity			Not Applicable

## GRANATECH

(ALL DERIVATIVES)

- STABILITY AND REACTIVITY

#### Stability

Stable

### Conditions to avoid

### None Known

#### Materials to avoid

None Known

### Hazardous decomposition products

None Known

- TOXICOLOGICAL INFORMATION

Toxicological information not available.

### HEALTH EFFECTS

#### Eye

Can cause burns

#### Skin

Causes irritation when wet.

#### Inhalation

Causes respiratory irritation.

#### Ingestion

Causes irritation of mouth, throat and digestive tract.

- ECOLOGICAL INFORMATION

Prevent contamination of soil, drains or surface water.

Mobility

Granular powder

Solubility

Forms aqueous mixture

Persistence & degradability

Once set, long term stable.

Bioaccumulative Potential

Once set, long term stable.

- DISPOSAL CONSIDERATIONS

For large quantities disposal must be carried out under guidance from local and national legislation.  Small quantities can be disposed of as common waste via a licensed waste contractor.

- TRANSPORT INFORMATION

Not classified as dangerous for transport.

- REGULATORY INFORMATION

#### Symbol

Xi

#### Contains

Portland Cements

#### Risk Phrases

R36/37/38 Irritating to eyes, respiratory system and skin.

R41 Risk of serious damage to eyes

#### Safety Phrases

S1/2 Keep locked up and out of reach of children

S22 Do not breathe dust

S24/25 Avoid contact with skin and eyes

S26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice.

S28 After contact with skin wash with plenty of water

S37/39 Wear suitable gloves and eye/face protection

- OTHER INFORMATION

These data sheets are given in connection with the product being used for the purposes outlined in the Product Technical Data Sheet, available from EasiPoint Marketing Ltd. on request.  Use of the product for other purposes may result in risks not given above.

If the product is to be used by a third party at work, it is the duty of the initial recipient to ensure that the third party is supplied with the data given above.

Employers have a duty to inform employees and others who may be effected of any hazards given in the data above and any precautions which should be adopted.

Users of the product should undertake their own assessment of work place risks as required by the Health and Safety Legislation.

Further copies of this Safety Data Sheet are available from EasiPoint Marketing Ltd.

Safety Data Sheet According to Directive 91/155/EEC  CHIP 96